# Lab 03 – Worksheet

| Name: M. Shees Abbas | ID: ma11212 | Section:T4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: addi x10, x0, 12 addi x11, x0, 12 jal x1, sum addi x11,x10, 0 li x10,1 ecall j Exit sum: add x10, x11,x10 jalr x0,0(x1) Exit: end: j end

Add screenshot of your results (register)

PC before and after procedure call: PC before and after procedure return: Terminal output from ecall: The final values of registers:

## Task 2

Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: li x10, 4 li x11, 3 li x12, 2 li x13, 1 li x18, 10 li x19, 20 li x20, 30 jal x1, leaf j Exit leaf: addi sp, sp, -12 sw x18, 0(sp) sw x19, 4(sp) sw x20, 8(sp) li x20, 0 add x18,x10,x11 add x19,x12,x13 sub x20, x18, x19 mv x10, x20 lw x20, 8(sp) lw x19, 4(sp) lw x18, 0(sp) addi sp, sp, 12 jalr x0, 0(x1) Exit: end: j end

Add screenshot of your results

PC before and after procedure call: Sp after storing registers in stack and after loading from stack: Memory values: Final register

## Task 3 Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: li a0, 0x100 # a0 = base address of v li a1, 0 # a1 = k = 0 # Populate only the two elements the swap uses li t0, 10 li t1, 20 sw t0, 0(a0) # v[0] = 10 sw t1, 4(a0) # v[1] = 20 jal ra, swap # call swap(v, k) lw s0, 0(a0) # s0 = v[0], should now be 20 lw s1, 4(a0) # s1 = v[1], should now be 10 j end swap: slli t1, a1, 2 # t1 = k * 4 (each long is 4 bytes) add t1, a0, t1 # t1 = address of v[k] lw t0, 0(t1) # temp = v[k] lw t2, 4(t1) # t2 = v[k+1] sw t2, 0(t1) # v[k] = v[k+1] sw t0, 4(t1) # v[k+1] = temp jalr x0, 0(ra) # return to caller end: j end

Add screenshots of your results

PC before and after function call Memory before and after function call Final register values

## Task 4 Provide appropriately commented codes

.text .globl main main: li a0, 0x200 # a0 = base address of x (destination) li a1, 0x100 # a1 = base address of y (source) # Populate y with the string "Hi!" and the null terminator li t0, 72 # 'H' li t1, 105 # 'i' li t2, 33 # '!' sb t0, 0(a1) # y[0] = 'H' sb t1, 1(a1) # y[1] = 'i' sb t2, 2(a1) # y[2] = '!' sb zero, 3(a1) # y[3] = '\0' jal ra, strcpy # call strcpy(x, y) lbu s0, 0(a0) # s0 = x[0], should be 72 ('H') lbu s1, 1(a0) # s1 = x[1], should be 105 ('i') lbu s2, 2(a0) # s2 = x[2], should be 33 ('!') lbu s3, 3(a0) # s3 = x[3], should be 0 ('\0') j end # done strcpy: li t0, 0 # i = 0 loop: add t1, t0, a1 # t1 = address of y[i] lbu t2, 0(t1) # t2 = y[i] add t3, t0, a0 # t3 = address of x[i] sb t2, 0(t3) # x[i] = y[i] beq t2, zero, done # if y[i] == '\0', exit loop addi t0, t0, 1 # i += 1 j loop done: jalr x0, 0(ra) end: j end

Add screenshots of your results

Memory value before and after function call After: Final register values:

# Lab 03 – Introduction to RISC V Assembly (Jumps and Returns)

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR5 Results | AR7 Report Submission and Git Upload |
| --- | --- | --- | --- |
| Task 1 | /10 | /05 | /10 & /10 |
| Task 2 | /10 | /10 |  |
| Task 3 | /10 | /10 |  |
| Task 4 | /15 | /10 |  |
| Total Points | /100 Points |  |  |
| CLO Mapped | CLO 2 |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |

Repo github: https://github.com/M-Shees-Abbas/CA-labs.git